# Cybersecurity Incident Report:

# Network Traffic Analysis

Part 1: Provide a summary of the problem found in the DNS and ICMP traffic log.

The UDP protocol reveals that: DNS query goes out via UDP from user IP to DNS Server at 203.0.113.2 This is based on the results of the network analysis, which show that the ICMP echo reply returned the error message: and receives ICMP “udp port 53 unreachable” The port noted in the error message is used for: Port 53 The most likely issue is: UDP port 53 is either closed or blocked. Destination host is either actively rejecting the connection, is blocked by firewall, or the DNS service is not running on that port.

Part 2: Explain your analysis of the data and provide at least one cause of the incident.

Time incident occurred: Approx 13:24 Explain how the IT team became aware of the incident: Multiple users reported unable to access client’s website Explain the actions taken by the IT department to investigate the incident: - IT analyst accessed the website and received same error - Analyst used tcpdump to monitor network traffic while attempting connection - Analyst analyzes the packet captures to determine the issue - Analyst observes that DNS queries sent to DNS resulted in port unreachable Note key findings of the IT department's investigation (i.e., details related to the port affected, DNS server, etc.): - DNS server’s IP is 203.0.113.2 - All DNS queries were sent over UDP port 53 (standard DNS port) - Port 53 unreachable - Because DNS resolution failed, the web browser couldn’t get the IP address for the website the client is attempting to access Note a likely cause of the incident: - DNS service down - Firewall rule blocking UDP traffic - Misconfiguration on the DNS server preventing it from listening on port 53